ORDINANCE NO. 2001-18

AN ORDINANCE OF THE TOWN OF TALTY, TEXAS, AMENDING ORDINANCE NO. 99-08 ZONING ORDINANCE ARTICLE 2 BY ADDING SECTION 105 TO ARTICLE 2.  AS HERETOFORE AMENDED, TO DEFINE AND PROVIDE FOR SPECIAL BUILDING AND CONSTRUCTION REQUIREMENTS AND MATERIALS FOR COMMERCIAL USE GROUPS (CN).  PROVIDING A REPEALER CLAUSE; PROVIDING A SEVERABILITY CLAUSE; AND PROVIDING AN EFFECTIVE DATE.

WHEREAS, the Planning, Zoning and Ordinance Commission of the Town of Talty and the governing body of the Town of Talty, in compliance with state laws with reference to amending the Comprehensive Zoning Ordinance, have given the requisite notice by publication and otherwise, and after holding due hearings and affording a full and fair hearing to all property owners generally, the governing body of the Town of Talty is of the opinion that said zoning ordinance should be amended as provided herein; Now, Therefore,

BE IT ORDAINED BY THE TOWN OF TALTY, TEXAS:

SECTION 1.  That the Comprehensive Zoning Ordinance No. 99-08 of the Town of Talty, Texas as heretofore amended, to add Section 2-105, without change or amendment to any other provision or subsection of Section 2, said subsection 2-105 to read as follows:

SECTION 2-105 BUILDING MATERIALS FOR COMMERCIAL BUSINESSES, COMMERCIAL USE GROUPS (CN)

Masonry Requirements:

- Masonry in commercial districts shall be limited to the following materials: brick, stone, tilt-up concrete with aggregate exposed, poured or cast in place concrete with an approved designed exterior, decorative haydite or concrete blocks or tile, glass curtain walls or glass blocks and stucco.  Masonry materials shall meet the following requirements:

  - The standards for acceptable masonry construction types are listed below:

      - Stone Material:  Masonry construction using stone material may consist of granite, marble, limestone, slate, river rock, and other hard and durable naturally occurring all weather stone.  Cut stone and dimensional stone techniques are acceptable.

      - Brick Material:  Brick material used for masonry construction shall be hard fired (kiln fired) clay or slate material which meets the latest version of ASTM standard C216, Standard Specification for Facing Brick (Solid Masonry Unit Made of Clay or Shale), and shall be Severe Weather (SW) grade, and Type FB or FBS or better.  Unfired or under fired clay, sand, or shale brick are not allowed.

      - Concrete Masonry Units:  Concrete masonry units used for masonry construction shall meet the latest version of the following applicable specifications: ASTM C90 Standard Specifications for Hollow Load Bearing Concrete Masonry Units; ASTM C145, Standard Specification for Hollow and Solid Non-load Bearing Units.  Concrete masonry units shall have an indented, hammered, split face or other similar approved architectural finish as approved by the Board of Alderpersons. Lightweight concrete block or cinder block construction is not acceptable as an exterior finish.

      - Concrete Panel Construction:  Concrete finish or Pre-cast panel (tilt wall construction shall be painted, fluted, exposed aggregate, or other approved architectural concrete finish.  Smooth or untextured concrete finishes are not acceptable.

- Construction Standards:  The standards and criteria contained within this section are deemed to be minimum standards and shall apply to all new, altered or repaired construction occurring within the Town of Talty.

  - The exterior walls of all buildings in these zoning districts shall be of solid masonry or masonry veneer construction covering at least 90% of the walls.  Plain haydite or concrete block outside walls must be veneered with another decorative masonry material.  Masonry shall be brick or stone.

  - No exterior wall of any building shall be constructed of wood or corrugated metal.

- Roofing Materials.

  - Roofing materials shall be of slate, metal or asphalt; no wood shingles may be utilized. Roofing must be certified to last thirty (30) years.  (1) Metal Roofing shall include profiled panels, deep ribbed panels and concealed fastener systems. Exterior finish shall be film laminated or baked on enamel painted to the manufacturer’s standards.

(a)The use of corrugated metal, plastic or fiberglass panels with continuous corrugations or with corrugations less than 1 ½” deep is prohibited.  Fiberglass panels may be used for skylights only and may constitute no more than five percent of the roof area.

(b) The use of galvanized, aluminum coated, zinc-Aluminum coated or unpainted external metal finish is prohibited with the following exceptions.

(1) Copper panels may be unpainted.

(2) Zinc coated steel standing seam roofing panels may be used only when placed directly over ½” wood decking.

SECTION 3.  That all provisions of the ordinances of the Town of Talty in conflict with the provisions of this ordinance be, and the same are hereby, repealed, and all other provisions of the ordinances of The Town of Talty not in conflict with the provisions of this ordinance shall remain in full force and effect.

SECTION 4.  That should any sentence, paragraph, subdivision, clause, phrase or section of this ordinance be adjudged or held to be unconstitutional, illegal or invalid, the same shall not affect the validity of this ordinance as a whole, or any part or provision thereof, other than the part so decided to be invalid, illegal or unconstitutional, and shall not affect the validity of this ordinance as a whole, or any part or provision thereof, other than the part so decided to be invalid, illegal or unconstitutional, and shall not affect the validity of the Comprehensive Zoning Ordinance as a whole.

SECTION 5.  This ordinance shall take effect immediately from and after its passage and the publication of the caption as the law and charter in such cases provide.

DULY PASSED by the Town Council of Talty, Texas, on the 19th day of June, 2001.

APPROVED:

Mayor

ATTEST:

Town Secretary